g. Environment System using your available components and no external resistor.

This system will monitor:

Noise (sound sensor)

Temperature (DHT11)

Light (LDR, no resistor)

…and trigger a buzzer if any readings are out of the normal range.

1. Components

Arduino Uno

Sound sensor

DHT11 sensor

LDR

Buzzer (or LED)

Jumper wires

Breadboard (optional)

2. Connection Guide

Component	Arduino Pin Connection	Notes

Sound Sensor	A 0	Analog output to A0

DHT11	D2	Data pin to D2, VCC → 5V, GND → GND

LDR	A1	One leg → 5V, Other leg → A1 + GND (no resistor)

Buzzer / LED	D3	Positive → D3, Negative → GND

Power connections:

All GND → Arduino GND

All VCC → Arduino 5V

Notes on LDR: Without a resistor, we read analog values from A1. Thresholds are set in code.

3. Arduino Code

#include "DHT.h"

#define DHTPIN 2

#define DHTTYPE DHT11

DHT dht(DHTPIN, DHTTYPE);

#define SOUND_PIN A0

#define LIGHT_PIN A1

#define BUZZER_PIN 3

// Thresholds (adjust if needed)

#define NOISE_THRESHOLD 600     // 0-1023 scale from sound sensor

#define TEMP_HIGH 26           // °C

#define TEMP_LOW 20            // °C

#define LIGHT_THRESHOLD 600    // 0-1023 scale from LDR

void setup() {

Serial.begin(9600);

dht.begin();

pinMode(BUZZER_PIN, OUTPUT);

}

void loop() {

// --- Read Sound ---

int soundLevel = analogRead(SOUND_PIN);

Serial.print("Sound: "); Serial.println(soundLevel);

// --- Read Temperature ---

float temp = dht.readTemperature();

Serial.print("Temp: "); Serial.println(temp);

// --- Read Light ---

int lightLevel = analogRead(LIGHT_PIN);

Serial.print("Light: "); Serial.println(lightLevel);

// --- Check thresholds ---

if (soundLevel > NOISE_THRESHOLD || temp > TEMP_HIGH || temp < TEMP_LOW || lightLevel < LIGHT_THRESHOLD) {

digitalWrite(BUZZER_PIN, HIGH); // Alert

} else {

digitalWrite(BUZZER_PIN, LOW);  // OK

}

delay(1000); // Update every second

}

4. How It Works

Arduino reads sound, temperature, and light every second.

If:

Noise > 600 → too loud

Temperature > 26°C or < 20°C → uncomfortable

Light < 600 → too dark

The buzzer turns ON to alert the teacher.

All readings are printed in the Serial Monitor for testin